Beam Analysis

(Demo version)

Program Beam Analysis is used for analyze moment and shear develops, reaction, reinforced steel for positive and negative moment and actual deflection of each span. Beam Analysis uses LOAD PATTERN method for find maximum moment and maximum reaction. It can find position and maximum value of positive moment. Result can be view, save to disk and print to printer.

----------------------------------------------------

------------- Beam Analysis Support ----------------

----------------------------------------------------

Full version, product updates and general support information is available at: http://www.civilsoft.tripod.com/

my email address: Poonsawatt@Hotmail.com

----------------------------------------------------

--------------- System Requirement -----------------

----------------------------------------------------

- Microcomputer AT 80286 or higher

- Dos 3.30 or higher and also run on Win9x system (Dosprompt)

- 640 kb Ram

- VGA Display with 256kb

- Math Coprocessor (Optional)

- Mouse and Hard Disk (Optional)

----------------------------------------------------

------------ How to use Beam Analysis --------------

----------------------------------------------------

Open file “BEAM.EXE”. You will see dialog box on screen. The dialog contains some data that want you to enter. When you start “BEAM.EXE”, you will see “Project Information” dialog. This dialog consists many of “Input Line Data”, Project Name for example. You should press [TAB] key to switch to others “Input Line Data” and enter data for it. There are many dialogs in each process then these are all dialogs you have to perceive.

- Input Project Information, that’s a header of report such as Project Name, Subject Name (Beam number), By (your name) and Date (auto enter date for you).

- Press [F2] to select output device, output device is a device that program use to print out or save report. There are three Output device you ca select to use.

  - Screen, open report on screen only

  - Screen & Save to disk, like screen but also save report file to disk (you have to enter your report file name).

  - Printer, report will print out to printer only not open file on screen or save to disk.

- Press [F3] to enter constant data, it’s constants of material (fc’ and fs) and some data for calculate deflection (% live load, duration).

- Press [F4] to enter Beam's cross section properties

- Program will automatically call these processes

  - Beam Information (press [F5]), it contain beam’s information such as

    - Amount of Spans (NS).

    - Amount of Groups (NG), each group is represent to beam that they are same length, inertia and same load.

    - If group is blank or zero or group is equal to amount of span (NS), it’s mean no group and one span is one group.

    - If group is not zero, blank and not equal to amount of span, you have to enter your group’s information.

      - Span’s number that correspond to current group number.

      - Loop until all spans are assigned to groups.

    - Span’s information for each group such as span length, cross section and moment of inertia. However you can use relative value for moment of inertia, instead of exact value, like 1.0 or 1.5 (time to other spans)

  - Load Information (press [F6])

    - Amount of loads applies on each group.

    - After you enter amount of loads, program will ask you to input load value such as

      - Load weight, direction down is positive.

      - Load start, left most position that load apply to beam for uniform (and part uniform) load. For point load it’s a position that point load applies to beam

      - Load cover, distance form load start for uniform (and part uniform) load, it’s zero or left it blank for point load.

      - Load type, Dead Load (D) or Live Load (I), it’s necessary for method of load pattern to find maximum moment, shear and reaction at support.

    - Loop until finish input other groups.

  - Cantilever characteristic moment (press [F7])

    - Amount of loads applies on left cantilever span.

    - Left cantilever span length.

    - Amount of load applies on right cantilever span.

    - Right cantilever span length.

    - Program will ask you for enter load data (like load’s information above) and loop until finish enters all load cases for left and right cantilever span.

    - You can left they all blank if it’s has not cantilever span.

- Program will automatic calculate (or press [F8]) and send result to output device you have selected.

- After reading report, you can change beam’s cross section properties by pressing [F4] to optimize beam’s section, shear, reinforced steel and deflection and then press [F8] to recalculate and report again.

- Press Alt-N to begin new beam.

----------------------------------------------------

--------------- Report Description -----------------

----------------------------------------------------

Please see sample report below.

Sheet______________________ Project : __________________________

Date : Wednesday 30-05-2001 Subject : __________________________

By   : Poonsawatt@Hotmail.com

##########################################################################

*****************************

>>> Structure Information <<<

******************************

Number of Span : 3

-------------------------------------------

Span No.    Span Length             Iz

-------------------------------------------

1           4.00                1.00

2           5.00                1.00

3           4.00                1.00

##########################################################################

*******************

LOADING INFORMATION

*******************

--------------------------------------------------------------------------

SPAN    LOAD WEIGHT     LOAD START     LOAD COVER   D.L. OR I.L.

--------------------------------------------------------------------------

1        1500.00           0.00           4.00       D  UNIFORM

2        1000.00           0.00           5.00       I  UNIFORM

2         500.00           3.00           0.00       D  POINT

3        1500.00           0.00           4.00       D  UNIFORM

###########################################################################

>>> Left cantilever  DEAD LOAD MOMENT : 125.00 kg-m

LIVE LOAD MOMENT : 0.00 kg-m

>>> Right cantilever DEAD LOAD MOMENT : 125.00 kg-m

LIVE LOAD MOMENT : 0.00 kg-m

###########################################################################

fc'                        :   210.00 ksc

fs                         :  1200.00 ksc

Beam Width                 :    20.00 cm

Beam Dept                  :    40.00 cm

Effective Dept             :    35.00 cm

Covering Compression Steel :     5.00 cm

Shear Resistance,Allow     :  2941.75 kg,13390.03 kg, Min ps =   4.08 cm2

Group #1 = 1,3

Group #2 = 2

*********************************

>>> Moment and shear envelope <<<

*********************************

===========================================================

Span #: 1 Length : 4.00 m Def : 0.24 cm L/360 : 1.11 cm

===========================================================

@ Sect  SHEAR   HOG MOM  HAst  HAsc   SAG MOM  SAst  SAsc

===========================================================

0.00    2733    -125   0.35   0.00       0   0.00   0.00

0.80    1533       0   0.00   0.00    1581   4.41   0.00

1.60     333       0   0.00   0.00    2327   6.49   0.00

2.40   -1207       0   0.00   0.00    2113   5.90   0.00

3.20   -2407    -147   0.41   0.00     940   2.62   0.00

4.00   -3607   -2553   7.12   0.00       0   0.00   0.00

MAX SPAN MOMENT = 2364 kg-m.  Ast = 6.60 cm2.  Asc = 0.00 cm2.

At distance from left support = 1.82 metres

===========================================================

Span #: 2 Length : 5.00 m Def : 0.07 cm L/360 : 1.39 cm

===========================================================

@ Sect  SHEAR   HOG MOM  HAst  HAsc   SAG MOM  SAst  SAsc

===========================================================

0.00    2691   -2553   7.12   0.00       0   0.00   0.00

1.00    1691   -1004   2.80   0.00       0   0.00   0.00

2.00     691    -813   2.27   0.00     829   2.31   0.00

3.00    -309    -622   1.74   0.00    1019   2.84   0.00

4.00   -1809    -931   2.60   0.00       0   0.00   0.00

5.00   -2809   -2599   7.25   0.00       0   0.00   0.00

MAX SPAN MOMENT = 1067 kg-m.  Ast = 2.98 cm2.  Asc = 0.00 cm2.

At distance from left support = 2.69 metres

===========================================================

Span #: 3 Length : 4.00 m Def : 0.23 cm L/360 : 1.11 cm

===========================================================

@ Sect  SHEAR   HOG MOM  HAst  HAsc   SAG MOM  SAst  SAsc

===========================================================

0.00    3619   -2599   7.25   0.00       0   0.00   0.00

0.80    2419    -184   0.51   0.00     903   2.52   0.00

1.60    1219       0   0.00   0.00    2086   5.82   0.00

2.40    -321       0   0.00   0.00    2309   6.44   0.00

3.20   -1521       0   0.00   0.00    1572   4.39   0.00

4.00   -2721    -125   0.35   0.00       0   0.00   0.00

MAX SPAN MOMENT = 2343 kg-m.  Ast = 6.54 cm2.  Asc = 0.00 cm2.

At distance from left support = 2.19 metres

############ REACTION ##############

RDL( 1) =       2983 kg  RLL( 1) =          0 kg  RTL( 1) =       2983 kg

RDL( 2) =       3458 kg  RLL( 2) =       2840 kg  RTL( 2) =       6298 kg

RDL( 3) =       3588 kg  RLL( 3) =       2840 kg  RTL( 3) =       6428 kg

RDL( 4) =       2971 kg  RLL( 4) =          0 kg  RTL( 4) =       2971 kg

This report was saved by choose Screen & Save to disk device. It contains many parts and it looks very simple. The descriptions of each section are following.

Header

This section shows about project information.

Structure Information

This section is about physical properties of beam such as

- Amount of span

- Span length of each span

- Moment of inertia for each span

Loading Information

This section shows about load characteristic that applies to each span and external moment (or moment of cantilever span).

Structure and Material Constant

This section shows you about materials’ strength and beam’s cross section. It also shows you some critical value for designation such as shear resistant of concrete, shear allowable of section and minimum %steel by calculate form cross section and strength of reinforced steel.

## Moment and Shear

This section shows results. There are many part of result in report you have to understand.

- Detail of groups. It describes relation between groups and span number.

- Moment and shear. This is the most important section of report because almost data of report show in this section. Beam Analysis will slice each span to 6 section and report on each section. It will report shear, hogging moment (negative moment), sagging moment (positive moment) and reinforced steel for all moment, including report max sagging moment at about middle span.

## Last section, Reaction

This section shows you all of reaction. Beam Analysis will report form left most support (no. 1). It will report reaction due to dead load only, max reaction due to live load only and max reaction due to total load (dead load + live load). It will show maximum value by reflect of load pattern.

----------------------------------------------------

---------- Demo version and Full Version -----------

----------------------------------------------------

Demo version and Full Version is closely similar but only amount of spans and amount of loads on span. Demo version can calculate maximum 3 continuous spans and 3 load cases on each group but 20 continuous spans and 20 load cases for Full version (1,800 Bath for Thai user and $50 for foreigner).

For more information, please Contact me at Poonsawatt@Hotmail.com